CASE STUDY ON DEMONSTRATION OF NETWORK COMMUNICATION MODELS (OSI / TCP-IP)

Aim

To understand and demonstrate how data travels through the OSI model and TCP/IP model using Cisco Packet Tracer.

Objective

- Understand the basic concepts of the OSI and TCP/IP network models.

- Create a simple network using two PCs and a switch in Cisco Packet Tracer.

- Configure IP addresses and verify communication between the PCs.

- Observe packet movement through the network using Simulation Mode.

- Identify the functions of the different OSI and TCP/IP layers.

Selected Network Application

Simple PC-to-PC Network Communication using Cisco Packet Tracer

Overview

The experiment demonstrates a basic network in which two PCs communicate through a 2960 switch. Cisco Packet Tracer is used to configure the devices, test connectivity, and observe packet movement. The communication is represented using both the seven-layer OSI model and the four-layer TCP/IP model.

- PC0 – Source device

- PC1 – Destination device

- 2960 Switch – Connects the two PCs

- Copper Straight-Through Cables – Physical connection

Network Requirements

Hardware / Network Devices

- Two PCs

- One Cisco 2960 switch

- Copper straight-through cables

Software

- Cisco Packet Tracer

Network Configuration

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 192.168.1.10 | 255.255.255.0 |
| PC1 | 192.168.1.20 | 255.255.255.0 |

Network arrangement: PC0 → Switch → PC1

Network Communication Model Analysis

| Model / Layer | Main Function | Packet Information | Role in Experiment |
| --- | --- | --- | --- |
| OSI – Application | User/application communication | Application data | Represents the message generated by the user |
| OSI – Presentation | Data format / encryption | Formatted data | Represents data presentation |
| OSI – Session | Manages communication sessions | Session information | Controls the communication session |
| OSI – Transport | Reliable delivery using TCP/UDP | Transport information | Manages data delivery |
| OSI – Network | IP addressing and routing | Source/Destination IP | Provides logical addressing |
| OSI – Data Link | Frames and MAC addressing | MAC addresses | Provides local network delivery |
| OSI – Physical | Signals and transmission medium | Bits/signals | Carries data through the cable |
| TCP/IP – Application | Application communication | HTTP/DNS/FTP-type application data | Combines OSI Application, Presentation and Session functions |
| TCP/IP – Transport | End-to-end transport | TCP/UDP information | Corresponds to OSI Transport |
| TCP/IP – Internet | Logical addressing | IP information | Corresponds to OSI Network |
| TCP/IP – Network Access | Local network transmission | Ethernet/MAC and physical signals | Combines OSI Data Link and Physical functions |

Why Simulation Mode is Best for Demonstration?

- It allows the packet to be observed step by step.

- Students can see the packet move from PC0 to the switch and then to PC1.

- Packet details such as source IP, destination IP, source MAC and destination MAC can be inspected.

- It clearly connects practical packet movement with the OSI and TCP/IP layers.

- It provides a simple visual method for understanding network communication.

Network Communication Process

Create the network: Place PC0, PC1 and a 2960 switch in Packet Tracer.

Connect the devices using copper straight-through cables.

Configure PC0 with IP address 192.168.1.10 and subnet mask 255.255.255.0.

Configure PC1 with IP address 192.168.1.20 and subnet mask 255.255.255.0.

Open Command Prompt on PC0 and ping 192.168.1.20.

Verify successful replies from PC1.

Change Packet Tracer from Realtime to Simulation Mode.

Use Add Simple PDU to send a packet from PC0 to PC1.

Use Capture/Forward to observe the packet travelling through the switch.

Inspect packet details and relate them to the OSI and TCP/IP layers.

SDLC-Style Experimental Phases

1. Requirement Analysis

- Two PCs are required as source and destination.

- A switch is required to connect the PCs.

- IP addresses and subnet masks must be configured.

- Packet movement must be observable.

2. Network Design

- Topology: PC0 → Switch → PC1

- PC0 IP: 192.168.1.10

- PC1 IP: 192.168.1.20

- Subnet Mask: 255.255.255.0

3. Configuration

- Place the network devices in Cisco Packet Tracer.

- Connect the devices.

- Assign the specified IP addresses.

4. Testing

- Use ping from PC0 to PC1.

- Use Simulation Mode to generate a Simple PDU.

- Use Capture/Forward to inspect packet movement.

Example Test Cases

| Test Case | Expected Result |
| --- | --- |
| Ping PC1 from PC0 | Reply received from 192.168.1.20 |
| Invalid / incorrect IP configuration | Communication should fail |
| Send Simple PDU from PC0 to PC1 | Packet reaches PC1 |
| Capture/Forward observation | Packet movement can be observed step by step |

Results

- PC0 successfully communicates with PC1 through the switch.

- The packet can be observed moving through the network in Simulation Mode.

- The experiment demonstrates how different layers add and process communication information.

- Source and destination IP addresses and MAC addresses can be inspected.

- The relationship between the OSI and TCP/IP models can be understood through the same communication process.

Conclusion

The case study demonstrates a simple network communication process using Cisco Packet Tracer and relates the practical packet flow to the OSI and TCP/IP models. The experiment shows how application data is processed through different layers, how IP and MAC addressing are used, and how packets travel from a source PC through a switch to a destination PC. Simulation Mode provides a clear practical method for understanding the operation of both network communication models.

Done by:-
Aaswin Muthu V
241201001
B.E Mechatronics(3rd yr)